Brief Description: Design a rocket by calculating which parts are needed to have a Mass Fraction that will launch.

Achievement Reqs/Full Descr:

An important part of working in space travel is to know if your rocket will fly! In this quest you will be learning how to calculate the Mass Fraction of a rocket. In short, the Mass Fraction is the mathematical calculation that gives rocket engineers a scientific prediction of the rocket’s success in launching and flying. Design a rocket by calculating which parts are needed to have a Mass Fraction that will launch.

This quest will require you to:

- Research and understand the importance of Mass Fraction;

- Learn about rocket fuel types;

- Calculate the Mass Fraction;

- And design a rocket that will launch based on the Mass Fraction.

Main Checkpoints:

| 1 star | 2 stars | 3 stars |
| --- | --- | --- |
| Design a rocket and calculate the Mass Fraction | Design a rocket that has a Mass Fraction between 0.76 and 0.82 | Explain if and why your rockets would be able launch. |

Bonus Checkpoints:

BONUS: Design your own rocket with your own masses that has a Mass Fraction between 0.75 and 0.85

Materials:

3,2,1 Liftoff worksheet

Information Sheet

Pencil

Final Product:

3,2,1 Liftoff worksheet

Information for Info Sheet:

______________________________________________________________________________

Vocabulary:

Thrust

Nozzle

Propellants

Fuel

Oxidizer

Newton’s First Law (Saved for other activity)

Newton’s Second Law (Saved for other activity)

Newton’s Third Law (Saved for other activity)

Mass

Mass Fraction

Payload

Newton’s First Law: (Saved for other activity)

An object at rest tends to stay at rest. An object in motion tends to stay in motion. Unless either are acted upon by an unbalanced force.

An unbalanced force is something which is not at rest or in the same motion as an object in motion.

Example: If you were to set a ball on level ground (no slope.) The ball would stay there. If you were to roll the ball, the ball would roll until it hits the wall (or another object.)

Newton’s Second Law: (Saved for other activity)

The second law is a math equation which contains three parts. Mass (m), Acceleration (a), and force (f). Mass is the weight of an object. Acceleration is the speed of an object. Force is both the weight and speed of an object.

F = ma

This means that the Force of an object, say the ball in the first example. Is equal to the Mass (or weight) multiplied by the Acceleration (or speed).

We measure the speed of the ball in meters per second (m/s). However, since the ball is speeding up as it moves further, it is measured in meters-per-second per second (m/s ²). In other words, the ball starts at rest (0 m/s), and the next second it speeds up to 1 m/s. The next second it speeds up to 2 m/s, and so on. This means after 5 seconds, the ball is moving at 5 m/s or meters per second. So to represent the increase in speed, we use m/s ². In this case, the ball would have been moving at 1 m/s ².

Force is measured in Newtons (N). 1 Newton is equal to 1kg x m/s ²

Example: Using the first example, the ball’s mass is roughly 1 kilo (kg). The speed that the ball is rolling is roughly 2 (m/s ²). If we plug these numbers into the equation, it would look like this:

Force of the ball = 1kg x 2 m/s ²

This means if the ball were to hit the wall, it would hit it with a force of 2 Newtons (N).

Newton’s Third Law (Saved for other activity)

Every action has an equal and opposite reaction.

Example: A plane moves forward as the engine pushes backwards. If you are swimming, when you push your hand backwards through the water, you move forward.

Rocket Propellants:

Rockets today use either a solid or liquid propellant. Propellant does not simply mean fuel. It means both fuel and an oxidizer. The fuel is that part which burns, but to burn the fuel needs oxygen. The oxidizer gives the oxygen to the fuel.

In solid rocket propellants, (which are dry to the touch,) have both the fuel and the oxidizer together. Usually, the fuel is a mixture of hydrogen compounds and carbon and the oxidizer is made of oxygen compounds.

In liquid rocket propellants, the fuel and oxidizer are kept in separate containers. Liquid propellants are made of gases which are cooled to a point where they turn into liquids. When the engine is fired, the fuel and the oxidizer are mixed in the engine by spraying each through nozzles at the same time. The fuel of a liquid propellant is usually kerosene or liquid hydrogen, and the oxidizer is liquid oxygen. When using a liquid-propellant, the engine becomes more complicated. Because the fuel and oxidizer need to be separate, the engine needs pumps and valves to move the liquid. This also makes the engine heavier.

Hybrid rockets use both liquid and solid propellants. By using a liquid oxidizer stored in a separate tank from the solid fuel, the engine only needs one system to direct the liquid. Hybrid rockets are normally easier and cheaper to make and weigh less than liquid rockets. Although the solid propellant rockets are simpler than the hybrid, the hybrids tend to have better performance.

Mass of Rocket:

The mass, or weight of the rocket, is important in creating a successful flight. For a rocket to leave the ground, and get to space, the engine must produce a thrust that is larger than the total mass of the rocket. So, it is important to make sure the weight of the payload (the items or people in the rocket), the weight of the rocket (engine, structure, tanks) and the weight of the propellants can be lifted by the force of the propellant. To find if a rocket will be successful in launching, engineers use the term Mass Fraction (MF).

A typical rocket’s total mass could be divided in the following way:

Of the total mass, 90% is the propellants; 6% is the rocket structure (engines, tanks, fins, etc.); and 4% is the payload (astronauts, satellites, living supplies).

The Rocket Equation:

Mass Fraction (MF) is found by the following formula:

MF = (Mass of Propellants)/(Total Mass)

Ideal MF should be about 0.80

The larger the MF number, the less payload a rocket can carry. The smaller the MF number, the less distance it can travel.

______________________________________________________________________________

Worksheet:

Propellant Choices:

| Propellant Type |  | Mass |
| --- | --- | --- |
| Liquid |  | 2,000,000 kg |
| Fuel | Oxidizer |  |
| Liquid Hydrogen | Liquid Oxygen |  |
| Solid |  | 1,600,000 kg |
| Fuel | Oxidizer |  |
| Hydrogen and Carbon Compounds | Oxygen Compound |  |
| Hybrid |  | 1,800,000 kg |
| Fuel | Oxidizer |  |
| Hydrogen and Carbon Compounds (Solid) | Liquid Oxygen |  |

Rocket Choices:

| Rocket |  |
| --- | --- |
| The Explorer |  |
| Payload | Structure |
| 200,000 kg | 250,000 kg |
| To Infinity and Beyond |  |
| Payload | Structure |
| 800,000kg | 1,200,000 kg |
| Intergalactic Voyager |  |
| Payload | Structure |
| 280,000 kg | 300,000 kg |

1a. Using the tables, choose a rocket and propellant combination. Calculate the Mass Fraction (MF) of the rocket.

1b. Will your rocket be able to launch? Explain Why:

2a. Using the tables, choose a rocket and propellant combination that has a Mass Fraction (MF) between 0.76 and 0.82

2b.  Will your rocket be able to launch? Explain why:

3a. Design your own rocket using the same formula, but with your own weight combinations. Aim to have your Mass Fraction (MF) between 0.75 and 0.85

3b. Explain your choices for each weight. Why are they important for making the rocket launch?